CVA6 Hardware Fuzzer

Implementation & Replication Guide

Group 7  |  EEE6323 VLSI II  |  University of Florida

Deep Bhanderi  ·  Atish Maragur  ·  Ke-Wei Tung

# 1. Project Overview

This framework implements coverage-guided hardware fuzzing for the CVA6 RISC-V processor. The goal is to generate smarter RV32IM instruction stimuli — guided by Xcelium coverage feedback — that explore more of the CVA6 RTL state space than random testing.

Three strategies are compared:

- Random: New random seed every iteration. Baseline only.

- Mutation-Only: Always mutate from corpus. Always save.

- Coverage-Guided: Mutate from corpus. Save seed only if Xcelium coverage metrics improve (delta > 0). Best strategy.

⚠  First goal: get the full pipeline working with random strategy. Then add mutation. Do not skip steps.

# 2. Prerequisites

| Tool / Requirement | Purpose |
| --- | --- |
| Cadence Xcelium 23.03+ | RTL simulation and UCDB coverage collection |
| Cadence IMC | Coverage metric extraction from UCDB |
| Python 3.9+ | Fuzzing engine, mutation, logging, plotting |
| matplotlib | Coverage plots (pip install matplotlib) |
| Git + internet | Clone CVA6 RTL from OpenHW GitHub |
| 32GB RAM recommended | CVA6 compile is memory-intensive |

# 3. Directory Setup

Create this directory structure before starting:

cva6_fuzzer/

├── fuzzer/

│   ├── main.py

│   ├── seed_generator.py

│   ├── mutation_engine.py

│   ├── input_formatter.py

│   ├── xcelium_runner.py

│   ├── coverage_analyzer.py

│   └── results_logger.py

├── tb/

│   └── testbench.sv

├── scripts/

│   ├── coverage.ccf

│   └── plot_results.py

├── rtl/            ← CVA6 cloned here

├── sim_work/       ← Xcelium outputs (auto-created)

└── results/        ← logs and plots (auto-created)

mkdir -p cva6_fuzzer/{fuzzer,tb,scripts,sim_work,results}

cd cva6_fuzzer

# 4. Week 1 — CVA6 RTL Setup & Xcelium Environment

Owner: Deep Bhanderi

Goal: CVA6 compiling in Xcelium, testbench boots from a hand-written hex file, coverage.ucdb produced.

## 4.1  Clone CVA6 RTL

⚠  Use this exact tag — the testbench and xrun commands are built for cv64a6_imafdc_sv39.

git clone https://github.com/openhwgroup/cva6.git rtl/cva6

cd rtl/cva6

git checkout cv64a6_imafdc_sv39

git submodule update --init --recursive

cd ../../

## 4.2  Edit PATHS in xcelium_runner.py

Open fuzzer/xcelium_runner.py and update the 4 paths at the top:

CVA6_RTL_PATH  = "/your/path/to/rtl/cva6/rtl"

XCELIUM_HOME   = "/your/cadence/xcelium/install"

IMC_HOME       = "/your/cadence/imc/install"

WORK_DIR       = "./sim_work"

## 4.3  Write a Test Hex File

Before the seed generator exists, create sim_input.hex manually to test Xcelium:

# sim_input.hex  (save in sim_work/)

# Word address @20000000 = byte address 0x80000000

@20000000

00000093

00000113

0000006F

## 4.4  Compile CVA6 + Testbench (compile_once)

⚠  This takes 5–15 minutes. Run once per session. Do not kill it early.

xrun \

tb/testbench.sv \

$(find rtl/cva6/rtl -name "*.sv" | tr "\n" " ") \

-elaborate \

-snapshot cva6_snapshot \

-xmlibdirname sim_work/xcelium.d \

-coverage all \

-covfile scripts/coverage.ccf \

-access +rwc -sv -timescale 1ns/1ps

Success: "Elaboration successful" in terminal. sim_work/xcelium.d/ directory is created.

## 4.5  Run One Simulation

xrun -R \

-snapshot cva6_snapshot \

-xmlibdirname sim_work/xcelium.d \

-defparam 'testbench.HEX_FILE="sim_work/sim_input.hex"' \

-defparam testbench.MAX_CYCLES=2000 \

-coverage all -covfile scripts/coverage.ccf \

-covdb sim_work/coverage.ucdb -exit

Week 1 complete when: simulation finishes, sim_work/coverage.ucdb exists.

# 5. Week 2 — Seed Generator, Formatter & Random Loop

Owner: Deep Bhanderi

Goal: seed_generator.py and input_formatter.py working, random strategy looping 10+ iterations without error.

## 5.1  Seed Structure

Every seed is a list of 30 integers (32-bit instructions):

- CI block (index 0–9): Context Initialization. Sets SP, clears registers, sets mtvec. NEVER mutated.

- TI block (index 10–29): Test Instructions. Random RV32IM. This is the mutation target.

## 5.2  Test seed_generator.py

cd fuzzer/

python -c "

from seed_generator import SeedGenerator

g = SeedGenerator()

s = g.generate()

print(f\"Generated {len(s)} instructions\")

print(f\"First (CI): {hex(s[0])}\")

print(f\"Last (TI):  {hex(s[-1])}\")

"

## 5.3  Test input_formatter.py

python -c "

from seed_generator import SeedGenerator

from input_formatter import write_hex

seed = SeedGenerator().generate()

write_hex(seed, \"../sim_work/sim_input.hex\")

print(\"Hex file written\")

"

## 5.4  Run Random Loop (10 iterations)

python main.py --strategy random --iterations 10

Week 2 complete when: 10 iterations run, 10 UCDB files produced, no errors.

# 6. Week 3 — IMC Coverage Extraction & Feedback Loop

Owner: Ke-Wei Tung

Goal: coverage_analyzer.py extracting all 6 metrics from UCDB, delta scoring and corpus update running end-to-end.

## 6.1  Test IMC Extraction Manually

First verify IMC works standalone on a UCDB from Week 2:

# Create extract_coverage.tcl

echo "load -run sim_work/coverage.ucdb" > test.tcl

echo "report -detail -metrics all -out sim_work/report.txt" >> test.tcl

echo "exit" >> test.tcl

# Run IMC

imc -exec test.tcl -quiet

# Check output

cat sim_work/report.txt | grep -i "coverage"

You should see lines like: "Statement Coverage: 87.3%" for each metric.

## 6.2  Test coverage_analyzer.py

python -c "

from coverage_analyzer import CoverageAnalyzer

a = CoverageAnalyzer(\"/your/imc/home\", \"../sim_work\")

m = a.extract(\"../sim_work/coverage.ucdb\")

print(m)

"

## 6.3  Weighted Metrics

| Metric | Weight |
| --- | --- |
| FSM | 3.0 |
| Branch | 2.0 |
| Condition | 2.0 |
| Expression | 2.0 |
| Toggle | 1.0 |
| Statement | 1.0 |

delta = sum( weight × max(0, new% − prev%) ) for each metric. Only positive improvements count.

## 6.4  Run Full Random Pipeline

python main.py --strategy random --iterations 50

Week 3 complete when: 50 iterations run, coverage metrics extracted each iteration, iterations.jsonl populated.

# 7. Week 4 — Mutation Engine

Owner: Atish Maragur

Goal: mutation_engine.py with 3 operators working, coverage-guided mode active.

## 7.1  Three Basic Operators

| Operator | What it does |
| --- | --- |
| bit_flip | Flip 1–3 bits in bits[31:7]. Opcode bits[6:0] never touched. Preserves instruction type. |
| byte_swap | Swap 2 non-opcode bytes within a 32-bit word. Changes register fields / immediates. |
| random_opcode | Replace opcode field with random 7-bit value. May create illegal instruction to exercise trap path. |

⚠  All operators apply only to TI block (index >= 10). CI block is never mutated.

## 7.2  Test mutation_engine.py

python -c "

from seed_generator import SeedGenerator

from mutation_engine import MutationEngine

seed = SeedGenerator().generate()

mut  = MutationEngine().mutate(seed.copy())

diffs = [(i, hex(seed[i]), hex(mut[i])) for i in range(len(seed)) if seed[i] != mut[i]]

print(f\"Changed {len(diffs)} instruction(s): {diffs}\")

"

## 7.3  Run Coverage-Guided Mode

python main.py --strategy coverage_guided --iterations 100

Week 4 complete when: coverage-guided mode runs 100 iterations, corpus grows over time, delta > 0 seeds are saved.

# 8. Week 5 — Full Pipeline Verification

Goal: all 3 strategies running cleanly. Verify stability before generating final results.

## 8.1  Run All 3 Strategies

python main.py --strategy random           --iterations 200

python main.py --strategy mutation_only    --iterations 200

python main.py --strategy coverage_guided  --iterations 200

⚠  Run each strategy separately. Results append to the same iterations.jsonl — strategy field distinguishes them.

## 8.2  Verification Checklist

- All 3 strategies complete 200 iterations without crash or timeout

- coverage.ucdb produced every iteration

- All 6 metrics extracted correctly each time

- Corpus grows over time in coverage-guided mode

- iterations.jsonl has 600 records total (200 per strategy)

- No zombie xrun processes (check: ps aux | grep xrun)

# 9. Week 6 — Results & Report

Owner: Ke-Wei Tung

## 9.1  Generate Plots

python scripts/plot_results.py \

--log results/iterations.jsonl \

--out results/comparison.png

Produces 4 subplots: cumulative delta, branch %, FSM %, corpus size — all 3 strategies overlaid.

## 9.2  Quick Reference — All Key Commands

| Command / File | Purpose |
| --- | --- |
| git submodule update --init --recursive | Initialize all CVA6 submodules after clone |
| xrun -elaborate ... | Compile CVA6 + testbench once (5–15 min) |
| xrun -R -snapshot ... | Run one simulation (1–5 sec) |
| imc -exec extract_coverage.tcl | Extract 6 coverage metrics from UCDB |
| python main.py --strategy random | Run random baseline strategy |
| python main.py --strategy coverage_guided | Run coverage-guided strategy |
| python scripts/plot_results.py | Generate comparison plots |

# 10. Troubleshooting

| Problem | Solution |
| --- | --- |
| xrun: command not found | Xcelium not in PATH. Add: export PATH=$XCELIUM_HOME/bin:$PATH |
| Submodule errors on clone | Run: git submodule update --init --recursive from inside cva6/ |
| $readmemh: file not found | HEX_FILE path must be absolute or relative to sim_work/ |
| coverage.ucdb not produced | Check xrun returncode. Often a hierarchy mismatch in testbench.sv |
| IMC shows 0% for all metrics | CCF scope paths may not match your testbench hierarchy. Check coverage.ccf |
| Compile takes > 30 min | Normal for first compile on slow machines. Use -xmlibdirname to cache |
| Testbench port mismatch | CVA6 port names vary by version. Check ariane.sv interface for exact names |
| Python import errors | Run from inside fuzzer/ directory or add fuzzer/ to PYTHONPATH |

# 11. Source File Reference

| File | Purpose |
| --- | --- |
| main.py | Top-level loop. Orchestrates all 3 strategies. Edit PATHS block at top. |
| seed_generator.py | Generates RV32IM bare-metal programs (CI + TI block). |
| mutation_engine.py | Applies bit-flip, byte-swap, random-opcode to TI block. Owner: Atish. |
| input_formatter.py | Writes $readmemh hex files. Boot addr 0x80000000 → @20000000. |
| xcelium_runner.py | compile_once() + run_simulation(). All xrun subprocess logic. |
| coverage_analyzer.py | IMC extraction, report parsing, weighted delta scoring. Owner: Ke-Wei. |
| results_logger.py | JSON-lines log per iteration + summary.json. |
| testbench.sv | CVA6 DUT + AXI memory model + 5 covergroups. |
| coverage.ccf | Xcelium coverage scope configuration. |
| plot_results.py | 4-subplot comparison graphs. Owner: Ke-Wei. |